PROTOFLOW MASTER PROMPT
1000W PORTABLE POWER STATION

Design a complete 1000W Portable Power Station as a professional editable KiCad project. Create the complete hierarchical schematic, PCB-ready design, BOM, footprints, manufacturing documentation, ERC/DRC-ready project, and engineering documentation. This is a commercial product prototype, not a simplified educational circuit.

1. PRODUCT SPECIFICATION

Product: 1000W Portable Power Station

Battery: 16S LiFePO4

Nominal Voltage: 51.2V

Maximum Voltage: 58.4V

Energy: Approximately 2000Wh

External 16S Smart BMS

AC Output:

1000W Continuous

Pure Sine Wave

- 230VAC

50Hz

Display:

2.4 Inch TFT LCD

ST7789 Controller

240×320 Resolution

SPI Interface

Display information:

Battery Voltage, Current, Power, SOC

PV Voltage, Current, Power

AC Output Voltage, Current, Power, Frequency

Temperature

Charging Status

Operating Mode

Fault Information

BMS Communication Status

Target switching frequency for high-frequency power stage: 100kHz

2. CREATE HIERARCHICAL KICAD SCHEMATIC

Create separate schematic sheets:

1. Power Tree

2. Battery Input & Protection

3. BMS Interface

4. PV Input Protection

5. MPPT Charger

6. Grid AC Input

7. AC Battery Charger

8. High Frequency DC/DC

9. HF Transformer

10. DC Bus

11. Pure Sine Inverter

12. AC Output Filter

13. AC Output Protection

14. Auxiliary Power Supply

15. MCU Control Board

16. Current Sensing

17. Voltage Sensing

18. Temperature Sensing

19. TFT Display

20. User Buttons

21. Fan Control

22. Communication

23. EMI EMC Filtering

24. Fault Protection

25. Debug & Programming

26. Test Points

Use professional net names and readable schematic organization.

Examples:

BAT+, BAT-, PV+, PV-, DC_BUS+, DC_BUS-, AC_IN_L, AC_IN_N, AC_OUT_L, AC_OUT_N, AUX_12V, AUX_5V, MCU_3V3, BMS_CAN_H, BMS_CAN_L, INV_PWM, MPPT_PWM.

3. BATTERY INPUT

Design for 16S LiFePO4 battery:

51.2V nominal

58.4V maximum

Include:

Battery Connector

Main Fuse

Reverse Protection

Pre-charge Circuit

Main Relay or Contactor

DC Bus Capacitors

Bleeder Resistor

Battery Voltage Measurement

Battery Current Measurement

TVS Protection

Hardware Overcurrent Protection

Undervoltage Protection

Overvoltage Protection

Temperature Input

Service Connector

Test Points

Expected continuous battery current at 1000W is approximately 20A, but design all power components with adequate transient and thermal margin.

4. EXTERNAL BMS INTERFACE

Create interface for Smart 16S BMS.

Support where available:

- UART

- CAN

Charge Enable

Discharge Enable

Fault Signal

Pack Voltage

Pack Current

- SOC

Temperature

Cell Fault

Include ESD protection, TVS, series resistors, pull-up/pull-down, isolation if required, connector and test points. Clearly separate BMS communication voltage levels from MCU logic.

5. PV MPPT CHARGER

Design PV charging section with:

PV Connector

Fuse

Reverse Polarity Protection

TVS Surge Protection

Common Mode Filter

Differential Mode Filter

Voltage Sensor

Current Sensor

MPPT DC/DC Converter

Gate Driver

Inductor

MOSFETs

Output Filter

Battery Charging Control

Hardware Protection

Temperature Sensor

If PV voltage range is not specified, define it as:

DESIGN PARAMETER – PV INPUT RANGE TO BE CONFIRMED

Do not invent unsafe voltage values. Use appropriate synchronous topology according to final PV range.

6. GRID AC INPUT & CHARGER

Create isolated AC charging system.

Include:

AC Connector

Fuse

- MOV

NTC Inrush Protection

EMI Filter

Common Mode Choke

Differential Filter

X Capacitor

Y Capacitors where appropriate

Rectifier

Isolated Charger

CC/CV Battery Charging

Voltage Feedback

Current Feedback

Temperature Protection

Output Protection

Relay Control

PE/Chassis connection where applicable

Maintain proper mains isolation from SELV circuitry and clearly identify hazardous voltage areas.

7. HIGH FREQUENCY DC/DC STAGE

Design isolated high-frequency converter around 100kHz.

Evaluate:

Full Bridge

Half Bridge

Push Pull

- LLC

Select the most suitable architecture and explain why.

Include:

Power MOSFETs

Gate Drivers

Gate Resistors

Pull-down Resistors

Bootstrap if required

Snubber

Current Sense

DC Link Capacitors

HF Transformer

Secondary Rectification

Output Capacitors

Feedback

Hardware Shutdown

For major components calculate/document:

Voltage Stress

RMS Current

Peak Current

Switching Loss

Conduction Loss

Ripple Current

Thermal Dissipation

Do not fabricate values without engineering justification.

8. HF TRANSFORMER

Create dedicated custom transformer specification:

Core Type

Core Material

Core Geometry

Effective Area

Primary Turns

Secondary Turns

Turns Ratio

Winding Method

Copper Type

Insulation

Creepage

Clearance

Flux Density

Core Loss

Copper Loss

Estimated Temperature Rise

Mark:

CUSTOM MAGNETIC COMPONENT – FINAL VERIFICATION REQUIRED

Do not invent manufacturer part numbers.

9. PURE SINE WAVE INVERTER

Design complete 1000W inverter:

- 230VAC

50Hz

Pure Sine Wave

Include:

DC Bus

MOSFET/IGBT Bridge

Gate Driver

SPWM Control

Dead Time

Current Sense

Voltage Sense

LC Output Filter

Output Relay

Fuse

Short Circuit Protection

Overload Protection

Overtemperature Protection

Emergency Shutdown

MCU must never drive power MOSFET gates directly. Use dedicated gate-driver circuitry.

10. AC OUTPUT FILTER

Design LC sine-wave filter and calculate:

Inductor

Capacitor

Resonant Frequency

Ripple Current

Saturation Margin

Damping Network

Document calculations.

11. AUXILIARY POWER

Generate:

- 12V

- 5V

- 3.3V

Power MCU, TFT, sensors, communication, gate drivers, fans and relays.

Include buck regulators, LDO where necessary, input protection, output filtering, decoupling, UVLO and test points.

12. MCU CONTROL BOARD

Select a suitable modern MCU with sufficient:

- PWM

- ADC

- SPI

- UART

- CAN

- I2C

Timers

- DMA

Watchdog

Debug Interface

MCU controls inverter, MPPT, charger, TFT, fan, relays, protection, buttons and BMS communication.

Include reset, boot circuit, SWD/JTAG, crystal if required, watchdog, brownout protection, decoupling, ESD and test points.

Explain MCU selection.

13. DISPLAY & USER INTERFACE

ST7789 SPI TFT, 240×320.

Signals:

VCC, GND, SCLK, MOSI, CS, DC, RESET, Backlight PWM

Create proper FPC connector interface with ESD and decoupling.

Buttons:

Power, Menu, Up, Down, Enter, Back, AC Enable

Provide debounce support and pull resistors.

14. MEASUREMENT SYSTEM

Current measurement:

Battery

- PV

AC Output

Charger

Voltage measurement:

Battery

- PV

DC Bus

AC Input

AC Output

Temperature:

Battery

MOSFET Heatsink

Transformer

Inductor

Charger

Ambient

Use suitable Hall or shunt sensing based on accuracy and isolation. Include RC filters and ADC protection.

15. FAN CONTROL

Design PWM fan controller with:

Automatic temperature control

PWM Speed

MOSFET Driver

Flyback Protection

Fan Connector

Optional Tachometer

Fan Fault Detection

16. EMI / EMC

Create dedicated EMI section including:

Common Mode Chokes

Differential Inductors

Ferrite Beads

X Capacitors

Y Capacitors

- TVS

Input Filters

Output Filters

Chassis Ground Strategy

Minimize high dv/dt switching loops and separate noisy power stage from sensitive MCU circuitry.

17. PROTECTION SYSTEM

Hardware:

Fast Overcurrent

Short Circuit

Gate Driver Shutdown

Battery OVP

Battery UVP

Thermal Shutdown

Emergency Stop

Software:

Overload

Overtemperature

Sensor Failure

Communication Failure

Fan Failure

Charging Fault

Inverter Fault

MCU failure must force the power stage into a safe OFF condition. Implement watchdog and fail-safe logic.

18. PCB REQUIREMENTS

Create professional 4-layer PCB.

Suggested stack:

Layer 1 – Components & High Current

Layer 2 – Ground Plane

Layer 3 – Power Distribution

Layer 4 – Signals & Components

Create functional zones:

A Battery Power

B MPPT

C Charger

D HF DC/DC

E Transformer

F Inverter

G AC Input

H AC Output

I MCU

J Display

K Sensors

L Communication

Keep hazardous voltage physically separated from SELV. Do not route MCU signals through high-power switching areas.

19. HIGH CURRENT PCB RULES

For every high-current net document:

RMS Current

Peak Current

Copper Thickness

Trace Width

Copper Pour

Via Quantity

Via Diameter

Temperature Rise

Use copper pours and multilayer parallel routing where required. Specify busbars if PCB copper is insufficient.

20. HIGH VOLTAGE SAFETY

Clearly define:

High Voltage Area

Low Voltage Area

Isolation Barrier

Creepage

Clearance

Isolation Slots

Keep-out Zones

Flag safety distances for final review according to applicable standards.

21. COMPONENT SELECTION

Use only real commercially available components. Preferred suppliers:

- LCSC

DigiKey

Mouser

For every critical component provide:

Manufacturer

- MPN

Package

Footprint

Voltage Rating

Current Rating

Power Rating

Temperature Rating

Datasheet Reference

Supplier

Critical parts include MOSFETs, gate drivers, MCU, relays, fuses, current sensors, TVS, capacitors, regulators, connectors and inductors.

If exact part cannot be verified, mark:

ENGINEERING PLACEHOLDER – REQUIRES DATASHEET VERIFICATION

Never invent part numbers.

22. CAPACITORS & THERMAL

Select capacitors according to voltage, ESR, ripple current, lifetime, temperature and switching frequency.

Perform first-order thermal calculation for MOSFETs, transformer, inductors, charger, inverter, relays and current sensors.

Specify heatsinks, thermal pads and fan cooling where required.

23. TEST POINTS

Include:

TP1 BAT+

TP2 BAT-

TP3 DC_BUS+

TP4 DC_BUS-

TP5 12V

TP6 5V

TP7 3V3

TP8 MCU_GND

TP9 PWM

TP10 Gate Supply

TP11 Battery Current

TP12 PV Voltage

TP13 PV Current

TP14 AC Voltage

TP15 AC Current

TP16 BMS UART

TP17 CAN

TP18 SPI

Add additional debugging points where necessary.

24. DESIGN RULES & DOCUMENTATION

Create KiCad design rules for:

Low Voltage

High Current

High Voltage

Isolation

Signal

Differential Pair

Generate:

Complete BOM

Component Placement Documentation

PCB Stackup

Power Tree Diagram

Test Procedure

Bring-up Procedure

Manufacturing Notes

Assembly Notes

ERC Report

DRC Report

Engineering Review Report

README must explain architecture and unresolved parameters.

25. FINAL DELIVERABLES

Export complete editable KiCad project containing:

.kicad_pro

.kicad_sch

.kicad_pcb

Custom Symbols

Custom Footprints

- BOM

Netlist

Manufacturing Files

Design Documentation

ERC Report

DRC Report

Test Point Documentation

Power Tree

Engineering Calculations

Final report must include:

Completed Blocks

Unverified Components

Unverified Calculations

Safety Critical Items

Thermal Concerns

EMI Concerns

PCB Concerns

Prototype Testing Checklist

Human Engineering Review Required

IMPORTANT ENGINEERING RULE

Do not claim the project is production-ready only because ERC/DRC passes. Every high-power, magnetic, thermal, isolation and safety-critical section must be documented and flagged for qualified power-electronics engineering review before fabrication.