TECHNICAL MANUAL: AUTOMOTIVE INJECTOR CALIBRATION DRIVER

Document Reference: MM-00017189-BUILD
Compliance Standard: Magneti Marelli 00017189 Rev.A.1
Target Parameters: Freq: 33 Hz | Peak Current: 4.0A | Hold Current: 2.0A | Pulse Width: 1–10 ms

1. SYSTEM BILL OF MATERIALS (BOM)

| Ref Designator | Component Name | Technical Specifications | Qty | Purpose |
| --- | --- | --- | --- | --- |
| U1 | NE555P | Bipolar Precision Timer IC (DIP-8 Package) | 1 | Base clock / pulse generation |
| U2 | LM1949N | Injector Drive Controller IC (DIP-8 Package) | 1 | Peak-to-hold sensing engine |
| U3 | LM7805CT | +5V Linear Voltage Regulator (TO-220) | 1 | Cleans logic power for U2 |
| Q1 | IRFZ44N | N-Channel Power MOSFET (55V, 49A, TO-220) | 1 | High-speed power switch |
| D1, D2 | 1N4148 | Fast Switching Diode (DO-35) | 2 | Separates 555 charge/discharge paths |
| D3 | 1N5364B | Zener Diode, 33V, 5W Power Rating | 1 | Absorbs inductive shutdown spikes |
| D4 | 1N4007 | Rectifier Diode, 1kV, 1A Rating | 1 | Blocks forward voltage into snubber |
| R1 | Fixed Resistor | 1.2 kΩ, 0.25W, 1% Metal Film | 1 | Safety limit for 555 Pin 7 |
| R_gate | Gate Resistor | 10 Ω, 0.25W, 5% Carbon Film | 1 | Dampens MOSFET switching oscillation |
| R_sense | Shunt Resistor | 0.1 Ω, 5W Low-Inductance Metal Strip | 1 | Senses current ramp-up thresholds |
| P1 | Potentiometer | 100 kΩ Linear Control Potentiometer | 1 | Frequency adjustment dial (10-50 Hz) |
| P2 | Potentiometer | 50 kΩ Linear Control Potentiometer | 1 | Pulse width adjustment dial (1-10 ms) |
| C1 | Electrolytic Cap | 2200 μF / 25V Radial Lead Aluminum | 1 | Filters 12V high-current bus sag |
| C2 | Film Capacitor | 0.47 μF (470 nF) Polyester Film | 1 | Main timing capacitor for U1 |
| C3 | Ceramic Cap | 0.01 μF (10 nF) 50V Ceramic Disc | 1 | Stabilizes internal reference of U1 |
| F1 | Fuse | 10A Automotive Blade Fuse + Inline Holder | 1 | System overcurrent safety backup |

2. ASSEMBLY STEP-BY-STEP INSTRUCTIONS

Step 1: Power Bus Assembly

- Mount the LM7805 (U3) on your board. Connect Pin 1 to the main incoming +12V fused bus. Wire Pin 2 straight to your primary Ground plane.

- Route a line from Pin 3 to create your clean +5V Logic Bus line.

- Install the 2200 μF capacitor (C1) across the main +12V track and the Ground wire. Place it physically close to where the fuel injector will plug in to smooth out heavy current surges.

Step 2: Build the NE555 Clock Controller

- Tie Pin 1 of the NE555 directly to Ground. Connect Pin 4 and Pin 8 to the +12V supply track.

- Solder the 1.2 kΩ resistor (R1) from +12V to the top pin of the P1 potentiometer. Tie the adjustable wiper of P1 directly to Pin 7.

- Place switching diodes D1 and D2 side-by-side facing opposite directions. This breaks the charging loop into two paths. Connect this array to P2, running the final control out to Pin 6 and Pin 2.

- Seat C2 (0.47 μF) between the Pin 6/2 node and Ground. Connect C3 (10 nF) from Pin 5 directly to Ground.

Step 3: Integrate the LM1949 and MOSFET Output Stage

- Set the LM1949 (U2) in place. Wire Pin 5 to your clean +5V Logic Bus. Wire Pin 2 to Ground.

- Route a jumper cable from NE555 Pin 3 (Output) directly to LM1949 Pin 4 (Input).

- Mount the IRFZ44N MOSFET (Q1) to an aluminum heatsink. Connect its Source pin to the high side of the 0.1 Ω 5W resistor (R_sense). Ground the lower leg of R_sense.

- Solder a line from LM1949 Pin 3 (Sense) directly to the node where the MOSFET Source meets R_sense.

- Connect LM1949 Pin 1 (Output) through the 10 Ω gate resistor to the Gate pin of the MOSFET.

- Install the high-voltage flyback clamp: Connect the 33V Zener (D3) anode to the MOSFET Drain pin. Connect its cathode to the anode of the 1N4007 (D4). Wire the cathode of D4 back to the +12V power supply rail.

- Connect the low side of your Fuel Injector to the MOSFET Drain pin and the high side to the +12V power supply rail.

3. CALIBRATION & TESTING PROTOCOL

Potentiometer Setup Lookup Matrix

Ensure your potentiometers are set accurately using a digital multimeter or oscilloscope tracking NE555 Pin 3:

- To Calibrate Base Frequency (33 Hz Target): Keep your scope on Pin 3 and turn P1 until the frequency matches exactly 33 Hz (this represents a cycle time of 30.3 ms).

- To Calibrate Target Injection Windows (\(T_{j}\)):

  - 1.0 ms Pulse Width: Set P2 to 1.86 kΩ (roughly 4% of its range).

  - 5.0 ms Pulse Width: Set P2 to 14.14 kΩ (roughly 28% of its range).

  - 10.0 ms Pulse Width: Set P2 to 29.49 kΩ (roughly 59% of its range).

Safe Operating Checks

- Double-check that a 10A fuse is secured inside the input cartridge.

- Flip on your 12.0V DC laboratory bench power supply.

- Hook up a current clamp or check the voltage across R_sense with an oscilloscope.

- Verify the waveform profile: The current must spike aggressively to 4.0A to unlock the low-pressure injector pintle, then immediately throttle down to a steady, low-ripple 2.0A hold current until the NE555 pulse signal shuts off.